ECE 3884 Lab 06: Code Optimization Fall 2008 Georgia Institute of Technology

1 Introduction

In the olden days, programming embedded devices meant programming directly in assembly language so that a programmer could scrutinize each instruction in order to optimize the program’s performance as measured by execution speed or code size. However, today’s modern compilers offer several optimization techniques for closing the gap between hand-written assembly language and compiled assembly language translated from a high-level language such as C. In this lab you will explore some of those optimization techniques and study their effects.

2 Lab Procedure

2.1 Research the Available Optimizations

- The IAR Embedded Workbench IDE offers several optimizations as part of its compiler. To choose which optimizations are performed, you must navigate to the Optimizations Menu by clicking on Project � Options � C/C++ Compiler � Optimizations.

- There you can select which optimization level you desire and customize which optimizations are performed.

- The IAR C/C++ Development Guide for ARM (available on the lab website) documents the available optimizations (see Table 19: Compiler optimization levels on page 160).

- Open the word document named Description of Optimizations Table.doc and fill in the table with a description of each optimization and their effects on execution time and code size. This document is included in the lab06.zip file.

- To expedite this process, we have provided the table on the next page with links to reference material for each optimization technique.

ECE 3884 Lab 06: Code Optimization Fall 2008 Georgia Institute of Technology

Table 1: Links to Reference Material for Each Optimization Technique

| Optimization Level | Optimization Description | References |
| --- | --- | --- |
| None | Variables live through their entire scope |  |
| Low | Variables only live for as long as they are needed |  |
| Medium | Live-dead analysis and optimization | See Computers as Components p. 271 |
| Medium | Dead code elimination | See Computers as Components p. 267 http://en.wikipedia.org/wiki/Dead_code_elimination |
| High | Redundant label elimination | http://tinyurl.com/redundant-label-elimination |
| High | Redundant branch elimination | http://tinyurl.com/Conditional-Branch-Elimination |
| High | Code hoisting | http://tinyurl.com/code-hoisting See Heiko Falk book p. 119 |
| High | Peephole optimization | http://en.wikipedia.org/wiki/Peephole_optimization http://tinyurl.com/peephole-optimization |
| High | Some register content analysis and optimization | See Computers as Components p. 270 http://en.wikipedia.org/wiki/Register_allocation |
| High | Static Clustering | See IAR C/C++ Development Guide for ARM p. 164 |
| High | Common subexpression elimination | See IAR C/C++ Development Guide for ARM p. 162 http://en.wikipedia.org/wiki/Common_subexpression_elimination |
| High | Instruction scheduling | See IAR C/C++ Development Guide for ARM p. 164 http://en.wikipedia.org/wiki/Instruction_scheduling |
| High | Cross jumping | http://tinyurl.com/Cross-Jumping |
| High | Advanced register content analysis and optimization | See Computers as Components p. 270 http://en.wikipedia.org/wiki/Register_allocation |
| High | Loop unrolling | See IAR C/C++ Development Guide for ARM p. 162 http://en.wikipedia.org/wiki/Loop_unwinding |
| High | Function inlining | See Computers as Components p. 267 See IAR C/C++ Development Guide for ARM p. 163 http://en.wikipedia.org/wiki/Inlining |
| High | Code motion | See IAR C/C++ Development Guide for ARM p. 163 http://en.wikipedia.org/wiki/Loop-invariant_code_motion |
| High | Type-based alias analysis | See IAR C/C++ Development Guide for ARM p. 163 http://en.wikipedia.org/wiki/Alias_analysis |

References for compiler optimization in general:

http://www.compileroptimizations.com/category/ http://en.wikipedia.org/wiki/Compiler_optimization

ECE 3884 Lab 06: Code Optimization Fall 2008 Georgia Institute of Technology

2.2 Experiment with Optimizations

For each of the following optimizations generate source code that individually exercises the specific optimization. Then enable just that optimization from the Optimization Menu. To observe the effect of the optimization, open the main.list list file that is generated by the compiler. IAR Embedded Workbench stores list files in the following directory:

<lab06>/ECE3884 Labs\at91sam7l-stk\lab06\ewp\at91sam7l128_flash\List

You will need to save a copy of the list file generated with no optimizations to compare against. Show a TA your results for each optimization and save your source code that exercises each optimization. You will need to turn in your source code and an explanation of how you exercised each optimization. To generate a new list file, perform a “Rebuild All” (Project Rebuild All).

1. Dead Code Elimination

TA Initial Lab06 Box 1

| 2. | Common Subexperession Elimination |  |
| --- | --- | --- |
|  |  | TA Initial Lab06 Box 2 |
| 3. | Loop Unrolling |  |
|  |  | TA Initial Lab06 Box 3 |
| 4. | Function Inlining |  |
|  |  | TA Initial Lab06 Box 4 |
| 5. | Code Motion* |  |
|  |  | TA Initial Lab06 Box 5 |

*Note: To exercise code motion, you may also need to enable Common Subexpression Elimination.

3 Lab Report

Your lab report should contain the following items:

- Description of Optimizations Table.doc filled in with descriptions of each optimization.

- Source code that individually exercises each optimization from Section 2.2.

- Description of how you exercised each optimization and the specific changes to the resulting assembly code.

- Answer the following questions: What risks are associated with using compiler optimizations? What are some downsides of using compiler optimizations?