# Average Calculator

## Objectives

Create a datapath/controller machine that computes the average of several numbers.

## Setup

This example is coded in Verilog. Set up your development environment.

## Procedure

Here is the Verilog for the datapath. The signal read_out is used for debugging visibility.

module average_calculator_dp(

input clk,

input [2:0] read0_adrs,

output [15:0] read_out,

input [2:0] read1_adrs,

input write_ena,

input [2:0] write_adrs,

input [15:0] write_val,

input alu_op,

input alu_write,

input alu_writeback,

output [15:0] write_out

);

reg [15:0] regfile [4:0];

reg [15:0] alutemp;

wire [15:0] read0_val, read1_val;

// register file

assign read0_val = regfile[read0_adrs];

assign read1_val = regfile[read1_adrs];

assign read_out = write_val;

assign write_out = alutemp;

always @(posedge clk)

begin

if (write_ena)

begin

regfile[write_adrs] <= write_val;

end

else if (alu_writeback)

begin

regfile[write_adrs] <= alutemp;

end

end

// ALU and temp register

always @(posedge clk)

begin

if (alu_write)

begin

if (alu_op)

begin

alutemp <= read0_val + read1_val;

end

else begin

alutemp[13:0] <= read0_val[15:2];

alutemp[15:14] <= 2'b00;

end

end

end

endmodule

Here is the Verilog for the controller.

module average_calculator_ctrl(

input clk,

input reset,

input go_load,

output reg done_load,

input go_calc,

output reg done_calc,

output reg write_ena, alu_op, alu_write, alu_writeback,

output reg [2:0] read0_adrs, read1_adrs, write_adrs

);

reg [3:0] state;

parameter idle = 4'd0,

load0 = 4'd1,

load1 = 4'd2,

load2 = 4'd3,

load3 = 4'd4,

op0a = 4'd5,

op0b = 4'd6,

op1a = 4'd7,

op1b = 4'd8,

op2a = 4'd9,

op2b = 4'd10,

openda = 4'd11,

opendb = 4'd12;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

state <= idle;

write_ena <= 1'b0;

alu_op <= 1'b0;

alu_write <= 1'b0;

alu_writeback <= 1'b0;

done_load <= 1'b0;

done_calc <= 1'b0;

end

else begin

case (state)

idle: // wait for command

begin

write_ena <= 1'b0;

read0_adrs <= 3'd0;

alu_write <= 1'b0;

alu_writeback <= 1'b0;

done_load <= 1'b0;

done_calc <= 1'b0;

if (go_load)

begin

write_ena = 1'b1;

write_adrs <= 3'd0;

state <= load0;

end

else if (go_calc)

begin

state <= op0a;

end

end

load0: // first of four loads

begin

write_ena <= 1'b1;

write_adrs <= 3'd0;

state <= load1;

end

load1:

begin

write_ena <= 1'b1;

write_adrs <= 3'd1;

state <= load2;

end

load2:

begin

write_ena <= 1'b1;

write_adrs <= 3'd2;

state <= load3;

end

load3:

begin

write_ena <= 1'b1;

write_adrs <= 3'd3;

done_load <= 1'b1;

state <= idle;

end

op0a: // first of pairwise adds

begin

read0_adrs = 3'd0;

read1_adrs = 3'd1;

alu_op = 1'b1;

alu_write = 1'b1;

alu_writeback = 1'b0;

state <= op0b;

end

op0b:

begin

alu_write = 1'b0;

alu_writeback = 1'b1;

write_adrs = 3'd4;

state <= op1a;

end

op1a:

begin

read0_adrs = 3'd4;

read1_adrs = 3'd2;

alu_op = 1'b1;

alu_write = 1'b1;

alu_writeback = 1'b0;

state <= op1b;

end

op1b:

begin

alu_write = 1'b0;

alu_writeback = 1'b1;

write_adrs = 3'd4;

state <= op2a;

end

op2a:

begin

read0_adrs = 3'd4;

read1_adrs = 3'd3;

alu_op = 1'b1;

alu_write = 1'b1;

alu_writeback = 1'b0;

state <= op2b;

end

op2b:

begin

alu_write = 1'b0;

alu_writeback = 1'b1;

write_adrs = 3'd4;

state <= openda;

end

openda:

begin

read0_adrs = 3'd4;

write_ena = 1'b0;

alu_op = 1'b0;

alu_write = 1'b1;

alu_writeback = 1'b0;

done_calc = 1'b0;

state <= opendb;

end

opendb:

begin

alu_write = 1'b0;

alu_writeback = 1'b1;

write_adrs = 3'd4;

done_calc = 1'b1;

state <= idle;

end

endcase

end

end

endmodule

Here is the Verilog for the complete average calculator.

module average_calculator(

input clk,

input reset,

input go_load,

output done_load,

input go_calc,

output done_calc,

input [15:0] val,

output [15:0] read_out,

output [15:0] write_out

);

wire [2:0] read0_adrs, read1_adrs, write_adrs;

wire write_ena, alu_op, alu_write, alu_writeback;

average_calculator_dp datapath(

.clk(clk),

.read0_adrs(read0_adrs),

.read_out(read_out),

.read1_adrs(read1_adrs),

.write_ena(write_ena),

.write_adrs(write_adrs),

.write_val(val),

.alu_op(alu_op),

.alu_write(alu_write),

.alu_writeback(alu_writeback),

.write_out(write_out)

);

average_calculator_ctrl controller(

.clk(clk),

.reset(reset),

.go_load(go_load),

.done_load(done_load),

.go_calc(go_calc),

.done_calc(done_calc),

.write_ena(write_ena),

.alu_op(alu_op),

.alu_write(alu_write),

.alu_writeback(alu_writeback),

.read0_adrs(read0_adrs),

.read1_adrs(read1_adrs),

.write_adrs(write_adrs)

);

endmodule

Here is a testbench for the machine.

module average_calculator_tb(

);

reg clk, reset, go_load, go_calc;

wire done_load, done_calc;

wire [15:0] read_out;

reg [15:0] val;

wire [15:0] op_result;

average_calculator average_calculator_under_test(

.clk(clk),

.reset(reset),

.go_load(go_load),

.done_load(done_load),

.go_calc(go_calc),

.done_calc(done_calc),

.val(val),

.read_out(read_out),

.write_out(op_result)

);

initial begin

clk = 1'b0; reset = 1'b1; go_load = 1'b0; go_calc = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; reset = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; reset = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; go_load = 1'b1; val = 16'd0;

#1 clk = 1'b1;

#1 clk = 1'b0; go_load = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; go_load = 1'b0;

#1 clk = 1'b1; val = 16'd2;

#1 clk = 1'b0;

#1 clk = 1'b1; val = 16'd9;

#1 clk = 1'b0;

#1 clk = 1'b1; val = 16'd1;

#1 clk = 1'b0;

#1 clk = 1'b1; val = 16'd8;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; go_calc = 1'b1;

#1 clk = 1'b1;

#1 clk = 1'b0; go_calc = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

end

endmodule

Here is the timing diagram produced by simulating the testbench.