# Combinational Logic

## Objectives

Synthesize and simulate combinational logic for addition.

## Setup

These examples are coded in Verilog. Install your synthesis and simulation tools if necessary.

## Procedure

A full adder takes three inputs and generates two outputs.

module fulladd(

input a,

input b,

input cin,

output sum,

output cout

);

assign sum = a ^ b ^ cin;

assign cout = (a && b) || (a && cin) || (b && cin);

endmodule

This testbench exercises the full adder.

module fulladd_testbench();

reg a, b, cin;

wire sum, cout;

fulladd a1(a,b,cin,sum,cout);

initial begin

$monitor("t = %d, a = %d, b = %d, cin = %d, sum = %d, cout = %d\n",

$time, a,b,cin,sum,cout);

a = 1'b0; b = 1'b0; cin = 1'b0;

#1 a = 1'b1; b = 1'b0; cin = 1'b0;

#1 a = 1'b0; b = 1'b1; cin = 1'b0;

#1 a = 1'b0; b = 1'b0; cin = 1'b1;

#1 a = 1'b1; b = 1'b1; cin = 1'b0;

#1 a = 1'b1; b = 1'b0; cin = 1'b1;

#1 a = 1'b0; b = 1'b1; cin = 1'b1;

#1 a = 1'b1; b = 1'b1; cin = 1'b1;

end

endmodule

Simulation produces this result.

A two bit adder is created from two full adders.

module two_bit_adder(

input a1, a0,

input b1, b0,

input cin,

output sum1, sum0,

output cout

);

wire c0out;

fulladd full1(a1,b1,c0out,sum1,cout);

fulladd full0(a0,b0,cin,sum0,c0out);

endmodule

This testbench exercises the two bit adder.

module tba_testbench();

reg a1, a0, b1, b0, cin;

wire sum1, sum0, cout;

two_bit_adder adder_under_test(a1,a0,b1,b0,cin,sum1,sum0,cout);

initial begin

$monitor("t = %d, a = [%d%d], b = [%d%d], cin = %d, sum = [%d%d], cout = %d\n",

$time, a1,a0,b1,b0,cin,sum1,sum0,cout);

a1 = 1'b0; a0 = 1'b0; b1 = 1'b0; b0 = 1'b0; cin = 1'b0;

#1 a1 = 1'b1; a0 = 1'b1; b1 = 1'b0; b0 = 1'b1; cin = 1'b0;

#1 a1 = 1'b0; a0 = 1'b0; b1 = 1'b1; b0 = 1'b1; cin = 1'b1;

#1 a1 = 1'b1; a0 = 1'b0; b1 = 1'b0; b0 = 1'b1; cin = 1'b0;

#1 a1 = 1'b0; a0 = 1'b1; b1 = 1'b1; b0 = 1'b0; cin = 1'b0;

#1 a1 = 1'b1; a0 = 1'b1; b1 = 1'b0; b0 = 1'b1; cin = 1'b1;

#1 a1 = 1'b0; a0 = 1'b0; b1 = 1'b1; b0 = 1'b1; cin = 1'b0;

#1 a1 = 1'b1; a0 = 1'b0; b1 = 1'b0; b0 = 1'b1; cin = 1'b1;

#1 a1 = 1'b0; a0 = 1'b1; b1 = 1'b1; b0 = 1'b1; cin = 1'b0;

#1 a1 = 1'b0; a0 = 1'b0; b1 = 1'b1; b0 = 1'b1; cin = 1'b0;

end

endmodule

Simulation produces this result.