# Four Cycle Handshake

## Objective

Analyze the design and operation of a four cycle handshake protocol.

## Setup

These examples are coded in Verilog. Install the logic synthesis and simulation environment of your choice.

## Procedure

The handshake is performed between an initiator and target. A testbench allows simulation of their operation.

Initiator:

`timescale 1ns / 1ps

module initiator(

output reg enq,

input response,

input ack,

input clk,

input reset,

input go

);

reg [1:0] state;

parameter idle = 2'b00,

request = 2'b01,

done = 2'b10;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

state <= idle;

enq <= 1'b0;

end

else begin

case (state)

idle:

begin

if (go)

begin

enq <= 1'b1;

state <= request;

end

end

request:

begin

if (ack)

begin

state <= done;

enq <= 1'b0;

end

end

done:

begin

if (!ack)

begin

state <= idle;

end

end

endcase

end

end

endmodule

Target:

`timescale 1ns / 1ps

module target(

input enq,

output reg response,

output reg ack,

input clk,

input reset

);

reg [1:0] state;

parameter idle = 2'b00,

respond = 2'b01,

finish = 2'b10;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

state <= idle;

ack <= 1'b0;

response <= 1'b0;

end

else begin

case (state)

idle:

begin

if (enq)

begin

ack <= 1'b1;

state <= respond;

end

end

respond:

begin

response <= 1'b1;

state <= finish;

end

finish:

begin

response <= 1'b0;

ack <= 1'b0;

state <= idle;

end

endcase

end

end

endmodule

Testbench:

`timescale 1ns / 100ps

module four_cycle_testbench(

);

reg clk, reset;

reg go;

wire enq, ack, response;

initiator initiator_under_test(enq,response,ack,clk,reset,go);

target target_under_test(enq,response,ack,clk,reset);

initial begin

$monitor("time $d: enq = %d, ack = %d, response = %d", $time, enq, ack, response);

clk = 1'b0; reset = 1'b0; go = 1'b0;

#1  reset = 1'b1;

#1  reset = 1'b0;

#1  clk = 1'b1;

go = 1'b1;

#1  clk = 1'b0;

go = 1'b0;

#1  clk = 1'b1;

#1  clk = 1'b0;

#1  clk = 1'b1;

#1  clk = 1'b0;

#1  clk = 1'b1;

#1  clk = 1'b0;

#1  clk = 1'b1;

#1  clk = 1'b0;

end

endmodule