# Pipeline Example

## Objectives

Simulate RISC-V code to observe the operation of the processor’s pipeline.

## Setup

This example uses the Ripes simulator https://ripes.me/.

## Experiment

This code sets up a taken branch:

.data

N: .word 4

a: .word 0

b: .word 0

c: .word 0

.text

lw x27, N

la x28, a

li x29, 1

lw x30, N

blt x29, x28, 12

addi x27, x27, -1

add x26, x26, x30

foo: sw x27, 0(x28)

add x28, x29, x27

add x30, x28, x29

Ripes demonstrates flushing of the not-taken instructions: