# Running Sum

## Objectives

Create a datapath/controller implementation of a machine that calculates a running sum of successive inputs.

## Setup

This example is coded in Verilog. Install your development environment.

## Procedure

Here is the Verilog for the datapath.

module running_sum_datapath(

input clk,

input resetacc,

input accval,

input [15:0] inval,

output [15:0] outval

);

reg [15:0] acc;

assign outval = acc;

always @(posedge clk)

begin

if (resetacc)

acc <= 16'b0;

else if (accval)

acc <= inval + acc;

end

endmodule

Here is the Verilog for the controller.

module running_sum_ctrl(

input clk,

input reset,

input go,

input done,

output reg resetacc,

output reg accval

);

parameter idlestate = 2'b01,

accum = 2'b10;

reg [1:0] state;

always @(posedge reset)

begin

state <= idlestate;

accval <= 1'b0;

resetacc <= 1'b0;

end

always @(posedge clk)

begin

case (state)

idlestate:

begin

accval = 1'b0;

if (go)

begin

state <= accum;

resetacc <= 1'b1;

end

else begin

state <= idlestate;

resetacc <= 1'b0;

end

end

accum:

begin

accval <= 1'b1;

resetacc <= 1'b0;

if (done)

state <= idlestate;

else

state <= accum;

end

default:

begin

state <= idlestate;

end

endcase

end

endmodule

Here is the Verilog for the full running sum unit.

module running_sum(

input clk,

input reset,

input go,

input done,

input [15:0] inval,

output [15:0] outval

);

wire resetacc, accval;

assign x1 = resetacc; assign x2 = accval;

running_sum_ctrl ctrl(clk,reset,go,done,resetacc,accval);

running_sum_datapath dp(clk,resetacc,accval,inval,outval);

endmodule

Here is a testbench.

module running_sum_tb(

);

reg clk, reset;

reg [15:0] inval;

reg go, done;

wire [15:0] outval;

running_sum running_sum_under_test(clk, reset, go, done, inval, outval);

initial begin

clk = 1'b1; reset = 1'b1; go = 1'b0; done = 1'b0; inval = 16'd0;

#1 clk = 1'b0; reset = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; go = 1'b1;

#1 clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0; go = 1'b0; inval = 16'd2;

#1 clk = 1'b1;

#1 clk = 1'b0; go = 1'b0; inval = 16'd4;

#1 clk = 1'b1;

#1 clk = 1'b0; go = 1'b0; inval = 16'd6;

#1 clk = 1'b1;

#1 clk = 1'b0; done = 1'b1; inval = 16'd0;

#1 clk = 1'b1;

#1 clk = 1'b0; done = 1'b0;

#1 clk = 1'b1;

#1 clk = 1'b0;

End

endmodule

Here is the timing diagram produced by simulation.