# Shift Register

## Objectives

Synthesize and simulate a shift register.

## Setup

This example is coded in Verilog. Setup your development environment.

## Procedure

Here is the Verilog for the shift register.

module shift_register(

input clk,

input reset,

input shiftin,

output reg shiftout

);

reg [1:0] bits;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

bits = 2'b00;

end

else begin

shiftout <= bits[1];

bits[1] <= bits[0];

bits[0] <= shiftin;

end

end

endmodule

Here is a testbench.

module shift_register_tb(

);

reg clk, reset;

reg shiftin;

wire shiftout;

shift_register shift_reg_under_test(clk, reset, shiftin, shiftout);

initial begin

reset = 1'b1; clk = 1'b0;

#1 clk = 1'b1; reset = 1'b0; shiftin = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; shiftin = 1'b0;

#1 clk = 1'b0;

end

endmodule

Here is the timing diagram produced by simulation.

Here is the schematic produced by synthesis.

## Extra

Modify the shift register to provide a parallel output of all the bits in the shift register.

Modify the shift register to produce a four-bit shift register.