# Simple Register

## Objectives

Synthesize and simulate an HDL model for a register.

## Setup

This example is coded in Verilog. Setup your development environment.

## Procedure

Here is the Verilog for the register.

module simple_register(

input clk,

input D,

output reg Q

);

always @(posedge clk)

begin

Q <= D;

end

endmodule

Here is a testbench.

module simple_reg_tb(

);

reg clk, D;

wire Q;

simple_register simple_reg_under_test(clk,D,Q);

initial begin

D = 1'b0; clk = 1'b1;

#1  clk = 1'b0;

#1  D = 1'b1; clk = 1'b1;

#1  clk = 1'b0;

#1  D = 1'b0; clk = 1'b1;

#1  clk = 1'b0;

end

endmodule

This is the timing diagram produced by simulation.

This is the schematic of the generated hardware.

## Extra

Modify the design to provide a two-bit register that reads both input bits at the same time.