# Two-Bit Counter

## Objectives

Synthesize and simulate a two-bit counter.

## Setup

This example is coded in Verilog. Setup your development environment.

## Procedure

Here is the Verilog description of the counter state machine as a register-transfer machine.

module two_bit_rt(

input clk,

input reset,

input incr,

output reg done

);

reg [1:0] state;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

state <= 2'b00;

done <= 1'b0;

end

else begin

state <= state;

done <= 1'b0;

if (incr)

begin

state <= state + 1'b1;

done <= (state == 2'b11);

end

end

end

endmodule

Here is a simple testbench for the counter.

module two_bit_counter_tb(

);

reg clk, reset;

reg incr;

wire done;

two_bit_rt tbc_under_test(clk, reset, incr, done);

initial begin

$monitor("time $d: reset = %d, incr = %d, done = %d\n", reset, incr, done);

reset = 1'b1;

#1 reset = 1'b0; clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

end

endmodule

Here is the timing diagram produced by simulation.

Here is the synthesized logic for the counter.

### FSM

Here is the Verilog description of the counter state machine as a finite-state machine.

module two_bit_counter(

input clk,

input reset,

input incr,

output reg done

);

reg [1:0] state;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

state <= 2'b00;

done <= 1'b0;

end

else begin

done <= 1'b0;

case (state)

2'b00:

if (incr)

state <= 2'b01;

else

state <= 2'b00;

2'b01:

if (incr)

state <= 2'b10;

else

state <= 2'b01;

2'b10:

if (incr)

state <= 2'b11;

else

state <= 2'b10;

2'b11:

if (incr)

begin

state <= 2'b00;

done <= 1'b1;

end

else

state <= 2'b11;

endcase

end

end

endmodule

Here is a simple testbench for the counter.

module two_bit_counter_tb(

);

reg clk, reset;

reg incr;

wire done;

two_bit_counter tbc_under_test(clk, reset, incr, done);

initial begin

$monitor("time $d: reset = %d, incr = %d, done = %d\n", reset, incr, done);

reset = 1'b1;

#1 reset = 1'b0; clk = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b0;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

#1 clk = 1'b0;

#1 clk = 1'b1; incr = 1'b1;

end

endmodule

Here is the timing diagram produced by simulation.

Here is the synthesized logic for the counter.

## Extra

Modify the design to count to three.

Create a three-bit design that counts to seven.