# Vending Machine

## Objectives

Build and simulate an FSM for a simple vending machine.

## Setup

This example is coded in Verilog. Install your development environment.

## Procedure

This module describes the encoded vending machine FSM.  Two primary outputs have been added for the internal state bits.

module vending_encoded(

input clk,

input reset,

input coin, product1, product2,

output reg vend1, vend2

);

reg [1:0] state;

parameter none = 2'b00,

one = 2'b01,

two = 2'b10;

always @(posedge clk or posedge reset)

begin

if (reset)

begin

state <= none;

vend1 <= 1'b0;

vend2 <= 1'b0;

end

else begin

case (state)

none:

begin

vend1 <= 1'b0;

vend2 <= 1'b0;

if (coin)

begin

state <= one;

end

end

one:

begin

vend1 <= 1'b0;

vend2 <= 1'b0;

if (coin)

begin

vend1 <= 1'b0;

vend2 <= 1'b0;

state <= two;

end

else if (!coin && product1)

begin

vend1 <= 1'b1;

vend2 <= 1'b0;

state <= none;

end

end

two:

begin

vend1 <= 1'b0;

vend2 <= 1'b0;

if (product2)

begin

vend1 <= 1'b0;

vend2 <= 1'b1;

state <= none;

end

else if (product1)

begin

vend1 <= 1'b1;

vend2 <= 1'b0;

state <= one;

end

end

endcase

end

end

endmodule

This is a simple testbench to exercise the machine.

module vending_machine_tb(

);

reg clk, reset;

reg coin, product1, product2;

wire vend1, vend2;

vending_encoded vending_machine_under_test(clk, reset, coin, product1, product2, vend1, vend2);

initial begin

$monitor("time $d: coin = %d, product1 = %d, product2 = %d, vend1 = %d, vend2 = %d\n", $time, coin, product1, product2, vend1, vend2);

reset = 1'b1;

#1    reset = 1'b0; clk = 1'b1;

#1    clk = 1'b0; coin = 1'b0; product1 = 1'b0; product2 = 1'b0;

#1    clk = 1'b1; product1 = 1'b1;

#1    clk = 1'b0; product1 = 1'b0;

#1    clk = 1'b1; coin = 1'b1;

#1    clk = 1'b0; coin = 1'b0;

#1    clk = 1'b1; coin = 1'b0;

#1    clk = 1'b0; coin = 1'b0;

#1    clk = 1'b1; product1 = 1'b1;

#1    clk = 1'b0; product1 = 1'b0;

#1    clk = 1'b1; coin = 1'b0;

#1    clk = 1'b0; coin = 1'b0;

#1    clk = 1'b1; coin = 1'b1;

#1    clk = 1'b0; coin = 1'b0;

#1    clk = 1'b1; coin = 1'b0;

#1    clk = 1'b0; coin = 1'b0;

#1    clk = 1'b1; coin = 1'b1;

#1    clk = 1'b0; coin = 1'b0;

#1    clk = 1'b1; product2 = 1'b1;

#1    clk = 1'b0; product2 = 1'b0;

#1    clk = 1'b1;

#1    clk = 1'b0;

end

endmodule

This is the timing diagram produced by simulating the testbench.

This is the schematic for the synthesized design.

## Extra

Change the FSM to a one hot encoding scheme.

Modify the vending machine and FSM to provide a change return option.