# Clap Detector

## Objectives

This lab builds a simple clap detector.  The detector matches a pattern to a decimated version of an audio signal.

This lab uses the Arduino Uno R4 WiFi and Velleman VMA309 microphone.

## Theory

The detector will run in the main Arduino loop() for simplicity. We will use the Arduino delay() function to set the sample rate. The sample rate will vary somewhat due to the computations performed in the main loop.

Samples are combined into a running sum for a fixed-size window. At the end of each window, the sum is divided by the number of samples to compute the average. Using a power-of-two window size allows us to perform integer division using a right shift. The average is then added to the decimation window circular buffer. A function tests whether the decimation window values correspond to a pattern consistent with a clap.

The threshold values used here are determined in part by the characteristics of audio capture. A different audio board could result in different thresholds.

## Setup

The analog output of the microphone is connected to an Arduino analog pin.

## Procedure

The setup function initializes the window running sum, the decimation circular buffer, and the builtin LED used to indicate a detected clap. We also initialize the serial port for possible use during debugging.

void setup() {

/* initialize */

Serial.begin(9600);

/* initialize running sum and decimation circular buffer */

int i;

isample = 0;

sum = 0;

for (i=0; i<NDECIMATE; i++)

decimate[i] = 0;

head = 0;

/* initialize LED */

pinMode(LED_BUILTIN,OUTPUT);

digitalWrite(LED_BUILTIN,0);

}

The dec_val() function returns the decimation buffer value at a given index.

int dec_val(int index) {

/* return ith value in circular decimation buffer */

int i;

i = (head + index) % NDECIMATE; /* add index to buffer head with modulo */

return decimate[i];

}

The add_decimate() function adds a new value to the decimation circular buffer.

void add_decimate(int val) {

/* add a value to the circular decimate buffer */

decimate[head++] = val;

if (head >= NDECIMATE)

head = 0; /* wrap around buffer */

}

The add_sample() function adds a value to the running sum.  If the window has been completed, the average calculation is completed by dividing the sum by the window size. Since a factor-of-two window size has been used, the division can be performed on the integer value using a right shift. The average is then added to the decimation buffer.

void add_sample(int val) {

/* add latest sample to running sum, update decimate circular buffer */

/* update sample buffer */

sum = sum + val; /* maintain running sum */

isample = isample + 1;

if (isample >= NSAMPLES) {

int result;

isample = 0; /* reset sample buffer */

/* divide by number of samples */

result = sum >> SAMPLES_POW2;

sum = 0;

/* add to circular buffer */

add_decimate(result);

}

}

The is_clap() function tests the decimation window values to see whether they conform to the clap pattern. The pattern is defined by two constants: THRESHOLD1 for the peak of the clap and THRESHOLD2 for the clap decay.

bool is_clap() {

/* test for clap decimation pattern */

int i;

return ( (dec_val(0) < THRESHOLD2) &&

(dec_val(1) >= THRESHOLD1) &&

(dec_val(2) >= THRESHOLD2) &&

(dec_val(3) < THRESHOLD2));

}

The Arduino loop() function reads the analog pin for the next sample, updates the running sum and decimation window, and lights the LED if the current window matches the clap pattern. It then uses the delay() function for the sampling period.

void loop() {

/* decimate, test, set LED */

int aval; /* analog read value */

/* sample */

aval = analogRead(A0);

// Serial.println(aval,DEC);

/* update buffers */

add_sample(aval);

/* test for clap signature */

digitalWrite(LED_BUILTIN,is_clap());

#if FALSE

if (is_clap()) { /* debug write */

int i;

Serial.print("detect = [ ");

for (i=0; i < NDECIMATE; i++) {

Serial.print(dec_val(i)); Serial.print(" ");

}

Serial.print(" ]\n");

}

#endif

/* wait */

delay(SAMPLING_INTERVAL);

}

}

## Extra

Change the audio sample rate.

Change the decimation factor and the associated clap filter values.

Modify is_clap() to compare the decimation buffer to an array of test values.

## Resources

Velleman VMA309 datasheet.