# Hall Effect Sensor

## Objectives

Build hardware and software for a Hall effect sensor.

## Setup

We use a prototyping board to assemble the Hall effect sensor circuit and its connections to the Arduino board.

The US5881 is a commonly used Hall effect device. A typical household magnet should be able to set off the sensor. Different versions of the device may react to north or south poles of a magnet. You can move the magnet in different orientations to find the one that causes the sensor to react. It has three terminals: supply voltage, ground, and output. The output pin requires a pullup resistor in the 10 kΩ range.

Initialization sets up both interrupts and the real-time clock. The real-time clock stops operation whenever power is removed. The clock must be set to the current time when clock power is restored.

We can write code to store Hall effect sensor activity in time/value event format. We use two arrays, one for time and another for the event value. Once the array is full, we wrap around to the start of the array, overwriting older values.

We toggle the board LED to indicate when the Hall effect device has been triggered.

Not all Arduinos provide a real-time clock. This example uses an Arduino Uno R4 WiFI board. The real-time clock needs to be initialized. This code initializes it to a standard time, allowing for testing relative to the power-up time for the board.  Other methods would need to be used to synchronize the real-time clock with a standard time reference.

#include <RTC.h>

#define HALL_EFFECT_PIN 2

const byte hall_effect_pin = 2;

int ievent = 0;

#define NEVENTS 16

RTCTime event_times[NEVENTS];

int led_state = 0;

void door_open_close_init() {

/* LED */

pinMode(LED_BUILTIN,OUTPUT);

/* Hall effect sensor */

pinMode(HALL_EFFECT_PIN,INPUT);

interrupts();

attachInterrupt(digitalPinToInterrupt(HALL_EFFECT_PIN),Hall_event_handler,FALLING);

/* real-time clock in Arduino UNO R4 WiFi; your mileage may vary */

RTC.begin(); /* start the clock */

RTCTime startTime(1,Month::JANUARY,2024,01,02,00,DayOfWeek::MONDAY,SaveLight::SAVING_TIME_INACTIVE); /* build time data structure */

RTC.setTime(startTime); /* set clock to desired value */

/* event log initialization */

ievent = 0;

}

An LCD shield can be attached to the board to print the time of the most recent event. This may require changing the assignment of the Hall effect pin, depending on what signals are passed through by the board.

## Procedure

The event handler reads the sensor and the real-time clock. It then stores the time/value pair in the event arrays. Separate initialization functions set up the real-time clock and the interrupt handler.

void Hall_event_handler() {

int sensorval;

RTCTime thistime;

sensorval = digitalRead(HALL_EFFECT_PIN); /* read the sensor */

RTC.getTime(thistime); /* get the time */

/* save in the event arrays, wrapping around when full */

event_times[ievent] = thistime;

ievent = ievent + 1;

if (ievent >= NEVENTS)

ievent = 0;

/* debug output */

led_state = !led_state;

digitalWrite(LED_BUILTIN,led_state);

}

## Extra

Add a display to the Arduino and use it to display the most recent event time.