# Micro Pong

## Objectives

Create a simple version of Pong that demonstrates display output reacting to button inputs.

## Setup

Install the display shield.

Create your micro_pong sketch.

Install the delay library.

Install the LiquidCrystal library.

## Procedure

Create your data structures, global variables, and setup function.

#include <LiquidCrystal.h>

#define TRUE 1

#define FALSE 0

#define LCD_LENGTH 16

#define LCD_HEIGHT 2

LiquidCrystal lcd(8,9,4,5,6,7);

void setup() {

// set up LCD

lcd.begin(LCD_LENGTH, LCD_HEIGHT);

lcd.setCursor(0,0);

}

Start with a simple test function to ensure that you can write to the screen.

void testfun() {

lcd.setCursor(0,0);

lcd.print("Pong me");

}

Build a function that can move the ball (the letter ‘O’) from one end of the screen to the other.

void bouncy_bouncy() {

int i;

lcd.clear();

while (TRUE) {

/* move from left to right */

for (i=0; i<LCD_LENGTH; i++) {

lcd.setCursor(i,0);

lcd.write('O');

delay(100);

lcd.setCursor(i,0);

lcd.write(' ');

}

}

The next step is to create a ball that bounces back and forth between the two ends of the screen.

void bouncy_bouncy() {

int i;

lcd.clear();

while (TRUE) {

/* move from left to right */

for (i=0; i<LCD_LENGTH; i++) {

lcd.setCursor(i,0);

lcd.write('O');

delay(100);

lcd.setCursor(i,0);

lcd.write(' ');

}

delay(200);

/* move from right to left */

for (i=LCD_LENGTH-1; i>=0; i--) {

lcd.setCursor(i,0);

lcd.write('O');

delay(100);

lcd.setCursor(i,0);

lcd.write(' ');

}

}

}

A more sophisticated version moves the ball back and forth between the two rows.

void up_down() {

int i, row = 0;

lcd.clear();

while (TRUE) {

/* check up/down buttons */

int button_in;

button_in = analogRead(0);

if (button_in <= 1000) {

if (button_in < 250) row = 0; /* up */

else if (button_in < 450) row = 1; /* down */

}

/* move from left to right */

for (i=0; i<LCD_LENGTH; i++) {

lcd.setCursor(i,row);

lcd.write('O');

delay(RATE);

lcd.setCursor(i,row);

lcd.write(' ');

}

delay(RATE);

/* move from right to left */

for (i=LCD_LENGTH-1; i>=0; i--) {

lcd.setCursor(i,row);

lcd.write('O');

delay(RATE);

lcd.setCursor(i,row);

lcd.write(' ');

}

}

}

We can build the full game as backboard().  Each execution of loop() calls backboard() once; each execution of backboard() performs one left-right-left traverse of the ball.

The buttons are connected to an analog input; the integer value returned by check_button() provides several ranges, each associated with a different button.

#define LCD_START 1

#define BUTTON_NONE 0

#define BUTTON_UP 1

#define BUTTON_DOWN 2

int check_button() {

/* check up/down buttons */

int button_in;

button_in = analogRead(0);

if (button_in >1000) return BUTTON_NONE; /* no push */

if (button_in < 250) return BUTTON_UP; /* up */

if (button_in < 450) return BUTTON_DOWN; /* down */

}

We will add a function that checks the button status and moves the paddle appropriately.

int paddle = 0, oldpaddle = 0;

void button_to_paddle() {

int buttonval = check_button();

if (buttonval == BUTTON_NONE) { /* don't change paddle row */ }

else if (buttonval == BUTTON_UP) { paddle = 0; }

else if (buttonval == BUTTON_DOWN) { paddle = 1; }

lcd.setCursor(0,oldpaddle); lcd.write(' '); /* remove old paddle */

lcd.setCursor(0,paddle); lcd.write('|'); /* write new paddle */

oldpaddle = paddle;

}

We will move some variables to globals so that they retain their state in between calls to the function. We will also introduce two forms of the ball: O for a regular ball and X for a ball that has been missed.

int row = 0, spin = 0;

#define BALL 'O'

#define MISSED_BALL 'X'

char ball = BALL;

The full function puts occasional spin on the ball by changing the ball’s row.  It also changes between the two ball symbols based on whether the paddle hit the ball.

void backboard() {

int i;

/* move from left to right */

for (i=LCD_START; i<LCD_LENGTH; i++) {

lcd.setCursor(i,row);

lcd.write(ball);

button_to_paddle();

delay(RATE);

lcd.setCursor(i,row);

lcd.write(' ');

}

delay(RATE);

/* occasional spin on bounce */

button_to_paddle();

spin++;

if (spin > 3) {

if (row == 0) row = 1; else row = 0;

spin = 0;

}

/* move from right to left */

for (i=LCD_LENGTH-1; i>=LCD_START; i--) {

lcd.setCursor(i,row);

lcd.write(ball);

button_to_paddle();

delay(RATE);

lcd.setCursor(i,row);

lcd.write(' ');

}

/* did paddle hit ball? */

if (row != paddle) ball = MISSED_BALL; else ball = BALL;

}

## Extra

Modify the game to provide beginner and tournament modes that operate at different ball speeds.

## Resources

LiquidCrystal - Arduino Reference

delay() - Arduino Reference