# Morse Code LED

## Objectives

- Understand basic concepts of C programs.

- Develop and understand a simple device driver.

- Be able to use features of an integrated development environment.

- Perform a program design cycle from specification to implementation.

## Setup

If Arduino IDE has not been installed on your host machine, install it now.

Use Sketch->Import Library to install the delay library. You will use delay() to perform wait operations:

delay(delaycount); // delay for delaycount ticks

Connect your board to your host system. Depending on your setup, you may or may not need a separate power cable.

You may need to use the Tools -> Board menu to select the type of board you are using.

## Procedure

Create a new sketch named blink_morse.

As you write your code, be sure to add comments that help you and others understand it. A good first step is a header comment:

/*

Blink_morse: blink a dit and a dah on the LED.

Written 1 January 2000 by Mary C. Programmer.

*/

Create the setup function.

void setup() {

// initialize LED as output

pinMode(LED_BUILTIN, OUTPUT);

}

Start the loop function by building an empty function.

void loop() {

}

Add code to blink the light for 500 ticks.

digitalWrite(LED_BUILTIN, HIGH);

delay(500);

digitalWrite(LED_BUILTIN, LOW);

Clock on Verify to compile and check your code. Click on Upload to upload the program to the Arduino.

Check the log window to check for possible errors and fix all errors.

You should see the LED blinking.

Add code to turn off the LED.

delay(500);

Add code to turn on the code for 1000 ticks and turn it off.

digitalWrite(LED_BUILTIN, HIGH);

delay(1000);

digitalWrite(LED_BUILTIN, LOW);

delay(500);

Add another space before the start of the next character.

delay(500);

## Extra

Use C constants to encode the long, short, and space delays.  You should be able to change every instance of each variable with a single edit.

Write code to display a pre-programmed sequence:

- Create an array int out[N] (you choose N to be a reasonable number). Each entry describes a dit, dah, or space.  Use 0 for space, 1 for dit, 2 for dah.

- Create a variable int outlen to hold the number of entries in out.

- Create code that will repeatedly play the sequence on the LED.

## Resources

delay() - Arduino Reference