# Smart Home

## Objectives

This lab builds a networked smart home system using MQTT.

## Setup

This system uses the Arduino Uno WiFiR2. This board does not provide a real-time clock.

You may need to install several libraries, including Arduino Mega AVRBoards, WiFiNINA, and ArduinoMqttClient.

This system requires two Arduino boards, one for the sensor node and another for the smart home console. This system was tested using separate computers to manage the two devices.

The system uses test.mosquitto.org, the test MQTT broker provided by Eclipse. Each project should set up its own message topic names in smart_home.h to avoid colliding with messages from another project.

Edit arduino_secrets.h to provide the name and password for your WiFi network.

## Procedure

The system uses two sketches, one for the door sensor and another for the console.  A common smart_home.h file provides shared declarations and includes.

The door sensor uses a modified version of the Hall effect sensor project.  This version uses polling since the MQTT service also makes use of polling.

The smart_home.h sensor includes strings for the message topics. Both the sensor and console need to use the same string for each topic.

#define MAX_MQTT_MESSAGE_SIZE 64 /* maximum allowed size of an element of an MQTT message */

const char door1_topic[] = "FoundationsOfComputerEngineering/smarthome/fdn_door1";

const char door2_topic[] = "FoundationsOfComputerEngineering/smarthome/fdn_door2";

### Door Sensor

The sensor initialization first connects to WiFi, then to the MQTT broker, and then subscribes to the topic appropriate for the node.  The include file gives topic names for two different doors so that the system can be tested with multiple doors.  The node can listen for its own messages to aid in debugging.

void mqtt_init() {

/* connect to network, connect to MQTT broker */

Serial.begin(9600);

while (!Serial) { } /* wait for serial port to connect */

/* connect to network */

Serial.print("Conneting to network...\n");

while (WiFi.begin(ssid,pass) != WL_CONNECTED) { }

/* connect to MQTT broker */

if (!mqttClient.connect(broker,port)) {

Serial.print("MQTT error "); Serial.println(mqttClient.connectError());

}

/* announce connection */

Serial.println("Connected to MQTT");

#if DEBUG

mqttClient.onMessage(onMqttMessage); /* listen for your own messages */

#endif

/* subscribe to topic related to all smart home subjects */

mqttClient.subscribe(door1_topic);

mqttClient.subscribe(door2_topic);

}

The Hall effect polling function reads the sensor pin and updates the status LED.  If the sensor has changed, it sends a message.

The main loop runs the sensor and MQTT polling functions.

void loop() {

/* main loop */

Hall_effect_poll(); /* test the Hall effect sensor, send MQTT message if sensor is activated */

mqttClient.poll(); /* poll the MQTT broker */

}

### Console

The console node listens for messages and prints them when received.  Its initialization is similar to that for the sensor node but it uses the onMessage method to tell the MQTT client system what function to call when a message is received. It also subscribes to all messages appropriate to the smart home.

void mqtt_init() {

Serial.begin(9600); /* set up serial port */

while (!Serial) { } /* wait for serial port to connect */

/* connect to network */

Serial.print("Conneting to network...\n");

while (WiFi.begin(ssid,pass) != WL_CONNECTED) { }

/* connect to MQTT broker */

if (!mqttClient.connect(broker,port)) {

Serial.print("MQTT error "); Serial.println(mqttClient.connectError());

}

/* announce connection */

Serial.println("Connected to MQTT");

/* set up message receive handler */

mqttClient.onMessage(onMqttMessage); /* handler */

/* subscribe to topic related to all smart home subjects */

mqttClient.subscribe(door1_topic);

mqttClient.subscribe(door2_topic);

}

The message handler reads the topic and value, then prints the message.

void onMqttMessage(int msgsize) {

/* receive and display a message */

String mytopic;

int myvalue;

/* receive message */

mytopic = mqttClient.messageTopic();

while (mqttClient.available()) {

myvalue = mqttClient.read() & 0x1;

}

if (mytopic == door1_topic) {

Serial.print("Door 1 = ");

Serial.print(myvalue);

}

if (mytopic == door2_topic) {

Serial.print("Door 2 = ");

Serial.print(myvalue);

}

Serial.print(".\n");

}

The sensor node is configured to first send messages for door2, then door 1.

The hub node receives the messages.

## Extra

Add timestamps from a real-time clock.

Add a database to store a history of messages.

## Resources

UNO WiFi Rev2 | Arduino Documentation

Sending Data over MQTT | Arduino Documentation

MQTT - Arduino Reference

Eclipse Mosquitto