# Water Bath Controller

## Objectives

Use a bang-bang controller to control the temperature of a water bath. This example sets the temperature thresholds as constants. The program could be modified to, for example, read the command temperature from a potentiometer on an analog port.

This example uses as a platform the Arduino Uno R4 WiFi. This project makes use of a heating element and water. Caution should be used in building and operating the setup. A 12V heating element is used to reduce the operating voltages but the setup still requires caution.

## Setup

The temperature sensor is a module that includes the thermistor and a DS18B20 programmable digital thermometer. The thermometer data wire is connected to digital pin D2 and requires a pullup pin. A heating element designed for automotive use is used here. This type of heating element operates at 12V. The heater command output is sent to a digital output pin D0 that controls a relay used to turn the heater on and off. This setup uses a separate lab power supply to power the heating element. The heater power supply must be able to supply the large current required by the heating element.

This lab uses the DS18B20 and OneWire libraries. Other libraries for this sensor may use different function names to provide similar functionality.

This program specifies the command temperature as a constant. A more sophisticated application would take the command from an external source.

Debug code allows us to test and debug our system. We can use the C preprocessor to include debug code when we need it and remove it from the application when it is no longer needed. This code uses two debug flags. DEBUG1 manages code that allows the control loop to be tested without sensors or actuators present. DEBUG2 manages code to observe the sensor.

DEBUG1 allows us to simulate the operation of the control system without using a sensor or actuator. When the simulated heater is off, the temperature drops one degree per period. When the simulated heater is on, the temperature increases by one degree per period. We can also change the starting temperature to test different starting points for the control system.

uint8_t temp_address[] = {40, 250, 31, 218, 4, 0, 0, 52};

uint8_t temp_selected;

DS18B20 ds(TEMPERATURE_PIN);

void setup() {

/* Setup the water bath temp controller. */

Serial.begin(9600); /* may be used for debugging */

pinMode(HEATER_PIN,OUTPUT); /* control for heater relay */

/* temp sensor setup */

temp_selected = ds.select(temp_address);

digitalWrite(LED_BUILTIN,0); /* initialize heater state on LED */

command = COMMAND_TEMP; /* extend this to read command from external source */

off_flag = (get_temp() <= command - EPSILON); /* should heater be on initially? */

#if DEBUG1

off_flag = (debug_temp >= command + EPSILON);

first = TRUE;

#endif

}

## Procedure

The get_temp() function reads the analog input pin for the temperature sensor. It also applies any scaling or other corrections necessary.

int get_temp() {

/* Read temperature from analog sensor. */

int aval;

ds.selectNext();

aval =  ds.getTempF();

#if DEBUG2

ds.selectNext(); Serial.print("sensor value = "); Serial.print(aval); Serial.print("\n");

#endif

return aval;

}

The bang_bang() function computes the control law. The allowable temperature range is specified by EPSILON.  It applies hysteresis using a Boolean off_flag to remember the state of the heater, which in turn determines which arm of the hysteresis loop is being followed.

bool bang_bang(int error) {

/* Determine heater state with hysteresis. */

bool heater;

if (error >= EPSILON) {

heater = FALSE;

off_flag = TRUE;

}

else if (error <= -EPSILON) {

heater = TRUE;

off_flag = FALSE;

}

else if (error > -EPSILON && off_flag) {

heater = FALSE;

}

else if (error < EPSILON && !off_flag) {

heater = TRUE;

}

return heater;

}

The loop() function performs step of the sampling interval.  It reads the temperature, computes the control law, and applies the result to the heater output pin. The result is also displayed on a built-in LED.

Debug code simulates the water bath behavior. The programmer can set the initial simulated temperature using debug_temp. If the heater is on, the temperature increases by one degree per iteration. If the heater is off, the temperature decreases by one degree per iteration. When operating properly, the temperature should cycle around the hysteresis loop with the heater turning on and off appropriately.

Here is loop() without the debug code.

void loop() {

/* Arduino main function. */

int current_temp, error;

bool heater;

current_temp = get_temp(); /* read temperature */

error = current_temp - command;

heater = bang_bang(error); /* analyze control law */

digitalWrite(HEATER_PIN,heater);  /* execute control action */

digitalWrite(LED_BUILTIN,heater); /* display heater state on LED */

delay(SAMPLING_INTERVAL); /* wait for next sample period */

}

Here is loop() with the simulated sensor and heater debug code.

void loop() {

/* Arduino main function. */

int current_temp, error;

bool heater;

/* simulate water bath temperature behavior */

if (first) {

current_temp = debug_temp; first = FALSE;

} else {

current_temp = debug_temp;

}

debug_temp = current_temp;

error = current_temp - command;

Serial.print("current temp = "); Serial.print(current_temp);

Serial.print(", error = "); Serial.print(error); Serial.print("\n");

heater = bang_bang(error); /* analyze control law */

if (heater) debug_temp = debug_temp + 1; else debug_temp = debug_temp - 1;

digitalWrite(HEATER_PIN,heater);  /* execute control action */

digitalWrite(LED_BUILTIN,heater); /* display heater state on LED */

delay(SAMPLING_INTERVAL); /* wait for next sample period */

}

## Extra

Modify the program to read the command temperature from a potentiometer connected to an analog input port. For even more control, add a second potentiometer to set the width of the hysteresis window.

Plot temperature vs. time for your water bath. You can write time/temperature pairs in CSV (comma-separated values) format that can be read by Excel and other plotting programs.

Evaluate the smallest EPSILON that your controller can maintain. What parameters can you adjust to reduce EPSILON?

Add a water pump that turns on and off with the heater to circulate water. Measure how the pump’s operation affects temperature vs. position in the tank.

## Resources

https://github.com/matmunk/DS18B20